Validation Session Note 7:

Sensor Calibration & UI Usability for Cross-generational Drivers

Date: [April 19, 2026]

User Profile: Aunt Liu, an elderly driver of shorter stature who shares a vehicle with her much taller son.

Target Demographic: Represents elderly users sensitive to hardware complexity and households with multiple drivers of varying physical profiles.

What We Tested (Scenario & Task)

This session evaluated the prototype’s performance when switching between drivers of different heights and the usability of physical buttons for parameter adjustment.

User Act: The user adjusted the seat forward to reach the pedals; the system then attempted to calibrate sensing distance and volume via physical hardware buttons.

Observations

Sensing False Positives: Because the user sat closer to the dashboard, the infrared sensor misinterpreted the proximity as "leaning forward due to drowsiness," triggering a persistent fatigue alarm immediately upon entry.

High Interaction Barrier: The "hold black button for 5 seconds" logic for settings was far too complicated for an elderly user, who reported being unable to handle the setup independently.

Multi-user Compatibility Issues: The alarm volume caused headaches. Furthermore, the fixed threshold failed to accommodate the drastic height difference between family members.

Changes Planned

Based on feedback regarding operational complexity and multi-driver limitations, we are adopting an IoT-based architecture:

APP-based Interaction: Integrate Wi-Fi/Bluetooth and replace complex physical buttons with a smartphone APP. Use large UI sliders for intuitive adjustment of "Fatigue detection distance threshold" and "Alarm volume".

Multi-driver Profiles: Implement cloud-saved profiles for different drivers (e.g., Aunt Liu vs. her son). This allows for one-tap threshold switching to solve the shared-vehicle friction.

Full Drive-cycle Optimization: Redefine the workflow from engine start and identity recognition to preset loading, ensuring a zero-false-alarm experience for users of all body types.

Evidence: Follow-up interview records via WeChat