Validation Session Note 3:

Multi-modal Alarm Interaction & Information Clarity

Date: [April 4, 2026]

User Profile: Wang Guo-hua, a 41-year-old driver who frequently commutes between Nanjing and Shanghai.

Target Demographic: This user represents the high-risk group of professional commuters who must drive long distances at night alone due to irregular work hours.

What We Tested (Scenario & Task)

This session evaluated the prototype's "single buzzer + OLED screen" alarm system during a real-world long-distance night drive.

User Act: When an alarm is triggered during the drive, the user attempts to identify the specific hazard (e.g., fatigue or speeding) using the provided hardware.

Observations

Ambiguous Feedback: The user reported that when the buzzer sounds halfway through a drive, they are unable to distinguish whether the alert is for fatigue driving or speeding.

Distraction Hazards: While the screen provides visual indications, looking at a small screen while driving at night is distracting and feels inherently unsafe.

Hardware Setup: The user confirmed the physical convenience of integrating the system onto the dashboard and the simplicity of the blue "start" button.

Changes Planned

Based on user feedback regarding the confusion and distraction caused by the current alarm, we are implementing a major upgrade to the interaction logic:

Modality Shift: Transition from a "single buzzer" to a multi-modal "Screen + Light/Sound + Voice" system.

Voice Integration: Introduce a dedicated voice broadcast module to replace the current ambiguous buzzer-only scheme.

"Combination" Logic: Implement a tiered reminder sequence where a high-frequency tone first catches the driver's attention, followed immediately by a clear voice broadcast specifying the hazard (e.g., "Drowsy driving detected" or "Speeding alert").

Eyes-free Interaction: Redesign the experience so that the user can accurately judge the danger type through hearing alone, eliminating the need to look at the screen and reducing driving risk.

Evidence: Follow-up interview records via WeChat