- 8050 Sound and Light Alarm Circuit + Software Development Flowchart

The alarm circuit uses a buzzer alarm circuit. The buzzer is similar in use to a speaker in household electrical appliances. Its operating current is usually relatively large, so the TTL level in the circuit cannot directly drive the buzzer. An additional current amplification circuit is therefore required. In other words, a single MCU pin can hardly drive the buzzer to sound, so a transistor is added to increase the current through the buzzer.

One positive terminal of the buzzer is connected to the 5 V power supply, and the other terminal is connected to the collector of the transistor. The base of the transistor is controlled by an MCU pin. When the corresponding MCU pin outputs a high level, the transistor turns on, the buzzer current forms a loop, and the buzzer sounds. When the pin outputs a low level, the transistor is cut off and the buzzer does not sound. To protect the circuit, a current-limiting resistor is added.

An LED is used as a common indicator. The LED, a current-limiting resistor, and the buzzer are connected in parallel. When the buzzer alarms, the LED lights up simultaneously. When the buzzer is silent, the LED is also off.

The flowchart is as follows:

Audio-visual alarm program entry

Alarm condition triggered?

No

Yes

MCU outputs a high level to activate the audio-visual alarm

Return