- 8050 Buzzer Alarm + LED + Circuit Explanation

An active buzzer is an integrated electronic sounder powered by DC voltage. It is widely used as a sound-producing device in computers, printers, copiers, alarms, electronic toys, automotive electronics, telephones, timers, and many other electronic products.

An LED, or light-emitting diode, is a type of semiconductor diode that converts electrical energy into light. Like an ordinary diode, it consists of a PN junction and also has one-way conductivity.

The alarm module used in this system is a 5V active buzzer module driven by an 8050 transistor. When the microcontroller control pin is at a high level, the 8050 turns on and the buzzer sounds; otherwise, it remains silent. The buzzer pattern can also be controlled by outputting a square wave from the microcontroller pin. The transistor amplifies current, and the resistor limits current. At the same time, an LED with a series resistor is connected in parallel with the buzzer. The series resistor acts as a current-limiting resistor. When the buzzer sounds intermittently, the LED flashes as an alarm indicator. The circuit is simple and reliable and does not require an additional microcontroller I/O pin.